- Section 102800 
Toilet, Bath, and Laundry Accessories - ASI

  - PART 1  GENERAL

    - SECTION INCLUDES

      - Healthcare accessories.

    - RELATED REQUIREMENTS

    - REFERENCE STANDARDS

    - SUBMITTALS

      - See Section 013000 - Administrative Requirements for submittal procedures.

      - Manufacturer's Installation Instructions:  Indicate special procedures and conditions requiring special attention.

  - PART 2  PRODUCTS

    - MANUFACTURERs

      - Basis of Design Manufacturer:  American Specialties, Inc:  www.americanspecialties.com/#sle.

      - Other Acceptable Manufacturers:

        - Products by listed manufacturers are subject to compliance with specified requirements found in specification and in Technical Data Sheet of selected ASI model and prior approval of Architect.

      - Substitutions:  See Section 016000 - Product Requirements.

        - All proposed substitutions (clearly delineated as such) must be submitted in writing for approval by Architect minimum of 10 working days prior to bid date and must be made available to all bidders. Proposed substitutes must be accompanied by review of  specification and ASI Technical Data Sheets noting compliance on line-by-line basis.

    - Healthcare Accessories

      - Turntable Specimen Pass Cabinet:

        - Turntable Cylinder Assembly:  18 gauge, 0.05 inch (1.27 mm) stainless steel, 12 inch (305 mm) diameter, with 10-1/2 inch (267 mm) by 6 inch (152 mm) opening; removable cylinder housing with lock; No.4 satin finish.

        - Cabinet Trim:  22 gauge, 0.0312 inch (0.79 mm) stainless steel, 1 inch (25 mm) flange with 1/4 inch (6 mm) returns, four sides; joints welded, sight-exposed welds finished to match sheet finish; adjustable for wall thickness 3 to 7 inches (76 to 178 mm).

        - Main Body:  Self-lubricating ball bearing plate mounted on top and bottom permits unit to revolve freely; positioned bullet catch prevents uncontrolled rotation.

        - Products:

          - Model 0515 - Specimen Pass Box (Turntable) - Recessed.

          - Substitutions:  See Section 016000 - Product Requirements.

    - Materials

      - Accessories - General:  Shop assembled, free of dents and scratches and packaged complete with anchors and fittings, steel anchor plates, adapters, and anchor components for installation.

- The paragraph below is optional text

    - Fabrication

    - Finishes

- The paragraph below is optional text

      - Shop Primed Ferrous Metals:  Pretreat and clean, spray apply one coat primer and bake.

- The paragraph below is optional text

      - Back paint components where contact is made with building finishes to prevent electrolysis.

  - PART 3  EXECUTION

    - EXAMINATION

      - Verify existing conditions before starting work.

      - Verify exact location of accessories for installation.

    - PREPARATION

      - Deliver inserts and rough-in frames to site for timely installation.

      - Provide templates and rough-in measurements as required.

    - INSTALLATION

      - Install accessories in accordance with manufacturers' instructions in locations indicated on drawings.

- The paragraph below is optional text

      - Mounting Heights:  As required by accessibility regulations, unless otherwise indicated.

        - Other Accessories:  As indicated on drawings.

    - PROTECTION

      - Protect installed accessories from damage due to subsequent construction operations.

- END OF SECTION